[HW4]

Smart Home

IoT 시스템 설계 01분반

21900699 조성준

2023.11.18

서론:

HW #4는 NodeMCU를 라스베리 파이를 통하여 실행되는 hass에서 다양한 정보를 표시할 수 있도록 하는 것을 목표로 한다. 핵심 기능으로는 사용자가 임의의 AP에 접속할 수 있도록 하는 것을 목표로 두고 있다. 그 외, HASS에 현재 위치를 표시하는 등, 여러 NodeMCU에서 읽어오는 센서 값들을 표시하도록 한다.

본론:

- WiFi 재설정

임의의 access point에 연결하기 위해서는 현재 연결된 클라이언트의 WiFi credentials을 수정 할 필요가 있다. 이를 하기 위해서는, EspMQTTClient 라이브러리의 setWifiCredentials 함수를 사용하였다. 그 후, 재접속을 유도하기 위해서, WiFi.disconnect(true) 함수를 사용하여 와이파이 연결을 끊고, EspMQTTClient가 재접속을 하도록 하였다.

StaticJsonDocument<200> doc; DeserializationError error = deserializeJson(doc,payload); if(error){ Serial.print(F("deserializeJson() failed: ")); Serial.println(error.f_str()); return; } WLANSSID = doc["SSID"]; WLANPW = doc["PW"]; Serial.println("\n\nChanges access point information: "); Serial.print("SSID: "); Serial.println(WLANSSID); Serial.print("PW: "); Serial.println(WLANPW); Serial.println(); // client.setWifiCredentials(const char *wifiSsid, const char *wifiPassword) client.setWifiCredentials(WLANSSID, WLANPW); WiFi.disconnect(true); MDNS.end();

입력 받는 새로운 AP의 정보는, iot/21900699/wifi 토픽에, json 형태로 받게 하였다. ArduinoJson 라이브러리를 사용하여 Json 안에 있는 내용을 parsing 하고, 필효한 정보만 가져와서 setWifiCredentials에 넘겨주는 방식을 사용하고 있다.

- HASS

HASS의 설정 파일인 configuration.yaml을 수정 하여, nth405의 온습도, 또한 개인 nodeMCU에서 읽어오는 값들을 표시 할 수 있도록 하였다. 추가로, 스위치를 사용하여 원격으로 간단하게 LED와 USBLED를 제어할 수 있도록 하엿다.

# Loads default set of integrations. Do not remove. default_config: homeassistant: name: joon latitude: 36.103295 longitude: 129.387009 elevation: 30 customize: "sensor.nth405_tempearture": icon: mdi:oil-temperature "sensor.nth405_humidity": icon: mdi:water-percent "sensor.nth405_light_intensity": icon: mdi:lightbulb lovelace: #mode: yaml #mode: storage mode: storage # Load frontend themes from the themes folder frontend: themes: !include_dir_merge_named themes # Text to speech tts: - platform: google_translate automation: !include automations.yaml script: !include scripts.yaml scene: !include scenes.yaml mqtt: sensor: - state_topic: "iot/nth405/dht22_t" name: nth405_temperature - state_topic: "iot/nth405/dht22_h" name: nth405_humidity - state_topic: "iot/nth405/cds" name: nth405_light_intensity - state_topic: "iot/21900699/dht_t" name: 21900699_temperature - state_topic: "iot/21900699/dht_h" name: 21900699_humidity - state_topic: "iot/21900699/CDS" name: 21900699_light_intensity switch: - name: "USB LED" state_topic: "iot/21900699" command_topic: "iot/21900699" qos: 0 payload_on: "usbledon" payload_off: "usbledoff" retain: false - name: "LED" state_topic: "iot/21900699" command_topic: "iot/21900699" qos: 0 payload_on: "ledon" payload_off: "ledoff" retain: false sensor: - platform: command_line name: CPU Temperature command: "cat /sys/class/thermal/thermal_zone0/temp" unit_of_measurement: "°C" value_template: "{{value ¦ multiply(0.001) ¦ round(1)}}" group: default_view: name: "NTH405" entities: - sensor.nth405_temperature - sensor.nth405_humidity - sensor.nth405_light_intensity 21900699: name: "21900699" entities: - sensor.21900699_temperature - sensor.21900699_humidity - sensor.21900699_light_intensity

이렇게 configuration.yaml을 설정 하고 실행 한 결과는, 3. 결과에서 확인할 수 있다.

- 결과

Home Assistant의 Overview (Dashboard) 화면

여러 센서를 통해 읽어오는 값들을 표시 하고, 스위치의 버튼을 사용하여 원격으로 LED와 USBLED를 제어할 수 있다.

다음 사진들을 통해 기본 상태, LED를 HASS를 통해 킨 상태, USBLED와 LED가 켜진 상태, USBLED만 켜진 상태를 확인할 수 있다.

다음 결과 사진(스크린샷)은 HASS에서 현재 위치를 확인할 수 있는 기능으로, HASS 대시보드에서 좌측에 있는 Map 탭을 이용하면 된다.

결론:

HASS를 사용하여, MQTT로 메시지를 주고받아 간단한 IoT 시스템을 설계 할 수 있는 것을 확인할 수 있었습니다. 원격으로 전등을 키고 끄는 것처럼 간단한 작업부터, 다양한 액추에이터 또는 센서를 사용하면, 만들 수 있는 시스템의 범위는 무궁무진 하다고 생각 합니다.

HASS는 이런 IoT 시스템을 여러 사람이 간단하게 구현할 수 있도록 좋은 플랫폼을 제공하고 있습니다.